Selective Electrochemiluminescent Sensing of Saccharides using Boronic Acids

Haidong Li,a Adam C. Sedgwick,b†  Meng. Li,b  Richard A. R. Blackburn,b Steven D. Bull,b Stéphane Arbault,a Tony D. James.b, Neso Sojic,a .

aUniversity of Bordeaux, ISM, UMR CNRS 5255, 16, Avenue Pey Berland, 33607 Pessac, France

b University of Bath, Department of Chemistry, Bath BA2 7AY, United Kingdom

† Haidong first author for contributing the most with the electrochemistry.

The discovery of electrogenerated chemiluminescence (ECL) in aqueous media has resulted from intense fundamental research on the mechanisms of the phenomenon1,2 and has led eventually to important bioanalytical applications including commercialized immunoassays for clinical diagnostics.3,4 The corresponding ECL process is based on a reaction cascade initiated at an electrode surface between a tandem system composed of a luminophore and a sacrificial co-reactant species. The luminophore reaches the excited state by the highly exergonic reactions with the electrogenerated co-reactant radicals. It relaxes then to the ground state by emitting ECL light which is the analytical signal. During the process, the luminophore is regenerated, whereas the co-reactant is consumed by the electrochemical reactions. The amount of light generated directly depends on the concentration of the luminophore but also of the coreactant. The luminophore has been used as ECL labels in numerous bioassays5-7 or as molecular probes with different receptor sites.8-13 For example, ruthenium and iridium complexes have been modified with crown ether moiety to measure different metal cations.8-11 In this case, the role of the coreactant is just to generate efficiently electrochemically suitable radicals with the adequate redox potentials and life-times. Another enticing possibility is to detect and to quantify any ECL-active co-reactant treated thus as an analyte as its concentration will directly influence ECL intensity. An example of the ECL detection of coreactants is the determination of oxalate and peroxydisulfate. Another class of analytes that may act as coreactants is amines. ECL assays for amine-based compounds find many analytical applications because amine groups are present in a large variety of pharmacologically important compounds such as antibiotics, anti-histamines, opiates, etc.7,14,15 However, a major limitation is that the analytes have to be electro-active and also able to generate strong reducing or oxidizing radicals to bring the luminophore to the excited state. So it limits strictly the nature of the possible molecules to detect.

As an alternative strategy, we propose to confer recognition properties to the coreactant by integrating a receptor site to its chemical structure. The recognition of the target analyte would then modulate its electrochemical properties and the resulting ECL emission of the luminophore. Here, we develop this completely new approach using a boronic acid as the receptor group. We chose the boronic acid receptor group as our first target due to its proven abillity to bind saccharides in water.16-22

Model systems providing high ECL efficiency in water consist of the luminophore Ru(bpy)32+ with co-reactants such as tri-n-propylamine (TPrA) or 2-(dibutylamino)ethanol.23-25 The pursuit of efficient ECL reagents is an intense area of investigation and several groups have studied the ECL mechanism implying amine coreactants.24,26-32 In general, tertiary amines produce ECL emission more efficiently than secondary amines and primary amines, following this order.7 Recently, it has been reported that increasing the electrochemical oxidation rate of amines leads to higher ECL efficiency. For instance, ECL emission is amplified with amines bearing electron-donating groups.25 In addition, aromatic amines, such as the pyridine ring of NAD+, the coenzyme of the dehydrogenase enzyme class, do not produce chemiluminescence or ECL with the ruthenium luminophore.33 By contrast, NADH, the reduced form of the coenzyme, may generate ECL emission because the aromaticity of the pyridine ring is destroyed and the aliphatic tertiary amine group undergoes ECL emission. Herein, inspired by the remarkable structure/reactivity relationship of this very efficient biomolecule, we designed and prepared a series of amine-based coreactants integrating boronic acid function as receptor units (Figure 1). For the first time, we demonstrated that the recognition of the saccharide modifies both the structure and the reactivity of the coreactant and thus the resulting ECL emission (Figure 1). With the presented approach, ECL generation depends on target molecules that are electro-inactive and do not to change the conformation of the luminophore as classically performed with molecular probes. (à developper) Moreover, differential selectivity for D-glucose and D-fructose is achieved by tuning the number of boronic acid groups and the spacer length. In addition, our approach would allow controlling the ECL readout signal with an additional parameter and not by playing only with the structure of the luminophore and the electrode potential. Therefore, we envision applications to biological and chemical sensing and molecular logic circuits.

Figure 1. - Chemical structures of the boronic acid based coreactants E0, E3 and E6.

Figure 2. b) Influence of the binding equilibrium of E0 to the saccharides (either D-fructose or D-glucose) on the ECL mechanism of the system Ru(bpy)32+/E0. Im+ is the iminium product.

To explore the usefulness of the above strategy, we selected the prototypical Ru(bpy)32+ complex as a luminophore and modified tertiary amines as oxidative-reduction coreactants because such a model system leads very efficiently to strong ECL emission.23 As a test case, we designed and prepared compound E0 which is similar to the TPrA structure but containing a boronic acid as recognition unit. E0 and the other co-reactants shown in figure 1 (E3 and E6) were synthesised through the alkylation of their corresponding amine using 2-(2-(bromomethyl)phenyl)-4,4,5,5-tetramethyl-1,3,2-dioxaborolane.

Reaction conditions – K2CO3 (3 eq), BPinBnBr (2 eq), amine, MeCN, 0 oC to rt, 16 h, n = 3 (92 %), n = 6 (16 %)

Once isolated, E0 electrochemical and ECL properties were investigated by using cyclic voltammetry in PBS solution that contained 10 µM of Ru(bpy)32+ and 0.2 mM of E0 (Figure 2). The coreactant concentration was in excess in comparison to the luminophore, as very classically employed in ECL experiments because it is consumed during the ECL process. As shown in Figure 2, irreversible oxidation of E0 occurred at 0.77 V vs. Ag/AgCl/KCl and the oxidation wave is shifted to less anodic potentials by 100 mV in comparison to the model TPrA reagent (Figure S1a). It indicates that the boronic acid group is a stronger electron-donor moiety compared to the simple propyl (have you observed similar behavior in your previous works? References?). This donor effect makes the molecule much easier to oxidize and can also stabilize the radical cation by charge delocalization.

Figure 3. Effect of glucose addition on electrochemical oxidation of E0 and on the corresponding ECL response. (a-b) Cyclic voltammetry and (c) ECL signal of a PBS solution (pH 7.4) containing 10 µM Ru(bpy)32+, 0.2 mM E0 and different concentrations of glucose. The arrow indicates increasing concentrations of glucose (0, 0.5, 1, 10, 50, 100 and 200 mM). Experiments were performed on glassy carbon (GC) electrode at a scan rate of 0.1 V s-1.

ECL emission occurred at the oxidation potential of Ru(bpy)32+ (Figure 2c), as for TPrA in the same experimental condition (i.e. low Ru(bpy)32+ concentration). It follows an oxidative-reduction mechanism where both the luminophore and the coreactant are oxidized (Figure 1b). After a deprotonation step, the cation radical gives a highly reducing neutral radical that react with the oxidized Ru(bpy)33+ to generate the excited state of the luminophore. This one relaxes in fine to the ground state by emitting ECL light. In comparison to TPrA, ECL response with E0 is divided by a factor 2 (Figure S1b). Indeed, the nature of substituents attached to nitrogen or α-carbon on the amine coreactant can also affect the ECL efficiency7,34 and it has been reported that aromatic-substituted amines that can conjugate the radical intermediates consistently lead to lower ECL emission response.35

Figure 4. a) Variation of the normalized ECL peak intensity (triangles) and of the oxidation peak current (dots) for the system Ru(bpy)32+/E0 as a function of the concentration of glucose (red markers) or of fructose (blue markers). b) Correlation between the ECL signals and the peak current corresponding to the E0 oxidation for the addition of different concentrations of D-glucose or of D-fructose. Values are extracted from Figures 2 and S2.

The addition of D-glucose caused a notable effect on both the voltammetric wave and the ECL response (Figure 2). It resulted in a dramatic decrease of the oxidation current along with its shift to more anodic potentials (Figure 2b). Correspondingly, ECL intensity decreases progressively with the addition of D-glucose. Under these conditions, the effects of D-fructose were qualitatively similar to the ones of D-glucose but with a much stronger decrease of both anodic wave and ECL intensity for a given concentration (Figure S1). Indeed, the oxidation wave almost vanished and the ECL peak decreased by 57% with 50 mM of D-fructose (Figure 3a). The same limit values for the ECL intensity and for the current were reached with 200 mM of D-glucose than with 50 mM of D-fructose. Such a difference was expected because the complexation constant of phenylboronic acid with D-fructose (4,400 dm3 mol-1) is much higher than with glucose (110 dm3 mol-1)18,36,37 To check that the observed decrease was related to the complexation of the saccharides by the phenylboronic acid from E0 and not to some photochemical or quenching effects, we recorded first the ECL spectra of Ru(bpy)32+ with increasing concentrations of D-fructose (Figure S3). Identical ECL and photoluminescent spectra were obtained showing that the photochemical properties of the luminophore were not affected neither by D-fructose nor D-glucose. Then we investigated the electrochemical and ECL signals of the model Ru(bpy)32+/TPrA in presence of D-fructose (Figure S4) and the effects of fructose addition led to minute changes. Finally, to determine the rate controlling step in the global ECL process, we plotted the variations of the normalized ECL peak (i.e. final readout signal) as a function of the normalized peak current (i.e. first step in the ECL process) with increasing concentration of D-fructose or D-glucose (Figure 3b). One can observe unambiguously that both signals are cross-correlated for both saccharides. It demonstrates thus that the variation of the ECL intensity is governed by the oxidation process of the coreactant. The electrochemical step, and more specifically the heterogeneous oxidation of E0, controls the efficiency of the ECL process. In other words, the decrease of the ECL intensity observed with increasing concentration of saccharides is due to the direct oxidation of the E0 at the electrode surface when it binds the saccharides.

The oxidation behavior of tertiary amines at glassy carbon electrodes has been examined in aqueous solutions.24,38-40 It is known that oxidation of a tertiary amine occurs when it is in the deprotonated form.24,29,30,41 When protonation of the tertiary amines becomes important at acidic pH, its oxidation peak decreases in intensity and shifts toward more positive potentials.24 As illustrated in Figure 1, the complexation of the saccharide by E0 induces a solvent-inserted B-N bond which behaves in a very similar fashion to protonation of the nitrogen atom.18,42,43 Therefore, heterogeneous oxidation of the coreactant in presence of saccharide becomes a very inefficient process that limits drastically the ECL signal. However, even at high D-fructose concentrations where almost all the E0 is bound, we still observe an ECL signal. Since ECL is obtained at the oxidation potential of Ru(bpy)32+, the remaining ECL intensity resulted probably from the electrocatalytic mechanism where E0 is homogeneously oxidized by the electrogenerated Ru(bpy)33+.

To further assess our approach and to improve the selectivity, we designed and prepared new symmetric tertiary bis amino reagents bearing boronic acid end-groups and having linkers with different lengths (Figure 1a). Previous, research has demonstrated that saccharide selectivity can be achieved in systems with two boronic acid groups.18,44-48 We investigated first the properties of the compound E6 with a hexyl linker between both amino groups. The cyclic voltammogram in Figure S1a is dominated by the irreversible oxidation waves of the amino moieties at the same potential than TPrA. The peak current is increased by approximately 2.1-fold indicating that both amino groups are simultaneously oxidized. However, ECL signal is 15% lower with E6 than with TPrA (Figure S1b), even if two amino groups were oxidized and may participate to the ECL process. This low ECL response is probably related to the lower stability of the electrogenerated dication radicals. The addition of fructose or glucose induces the decrease of the oxidation current and the shift of the corresponding anodic wave, as reported for E0 (Figure S5). Which resulted in a decrease of the ECL response in the presence of both saccharides (Figures S5 and S6). To further investigate the properties of E6, additional competitive assays were performed. They clearly showed that D-fructose and D-glucose caused notable modification of the current and of the ECl signal (Figure S7). Initially, it was somewhat surprising to us that E6 could not discriminate between D-fructose and D-glucose and detect them selectively.  However, from our previous fluorescence work46 for F6 the measured binding constants for D-fructose and D-glucose are very similar 784±44 and 962±70  dm3 mol-1

Figure 5. Effect of (a-b) glucose or (c-d) fructose addition on electrochemical oxidation of E3 and on the corresponding ECL responses. Cyclic voltammograms and ECL signals of a PBS solution (pH 7.4) containing 10 µM Ru(bpy)32+, 0.1 mM E3 and different concentrations of glucose or of fructose. The arrow indicates increasing concentrations of glucose or of fructose (0, 0.5, 1, 10 and 50 mM). Experiments were performed on glassy carbon (GC) electrode at a scan rate of 0.1 V s-1.

Further electrochemical experiments were performed on this coreactant structure to investigate the influence of the linker length on the selective recognition of both saccharides. Figure 4a presents the results obtained with E3 where the linker between both amino group and the phenylboronic acid moieties is shorter. The cyclic voltammogram showed that the oxidation process of E3 is more complex exhibiting a first anodic wave at 0.87 V vs. Ag/AgCl/KCl and a second at 0.95 V vs. Ag/AgCl/KCl (Figure 4a).

Figure 6. Variation of the ECL peak intensity for the system Ru(bpy)32+/E3 as a function of the concentration of glucose or of fructose.

The fact that both waves are partially separated indicates that the electronic communication between both amino groups is not negligible, in contrast to what was observed for E6. Adding D-glucose induces the decrease of the current for both oxidation waves as well as the parallel decrease of the ECL response (Figure 4b). Remarkably, a characteristic stability of the voltammetric signal and of the ECL response (Figure 4 c-d) was found for D-fructose, allowing discriminating between both saccharides (Figure 5). While, this result seems quite strange, our observations with similar fluorescence systems46 can help explain the observed selectivity. Since, the binding of F3 with D-fructose is significantly impaired when compared with the model system F0 evidenced by a 4-fold reduction in binding for D-fructose from 395±11 dm3 mol-1 for F0 to 95±9 dm3 mol-1 for F3. However, for D-glucose the binding is enhanced 2-fold from 44±3 dm3 mol-1 for X0 to 103±3 dm3 mol-1 for F3.

Figure 7 – Fluorescent boronic based saccharide sensors F0 and F3

The proportional decrease of the ECL readout signal with the D-glucose concentration allowed for facile quantification. Finally, the correlation of the ECL variation with the current for E3 confirmed that the oxidation step of the coreactant is the one determining the efficiency of the ECL process.

Conclusion paragraph

In summary we have developed a saccharide selective electrogenerated chemiluminescence (ECL) system that functions in aqueous media. We achieved excellent D-glucose selectivity by varying the linker length of a bis-boronic acid amine coreactant. The simplicity of the system ensures that it will find many applications from biological and chemical sensing to incorporation in molecular logic circuits.

Experimental section

N1,N3-dimethyl-N1,N3-bis(2-(4,4,5,5-tetramethyl-1,3,2-dioxaborolan-2-yl)benzyl)propane-1,3-diamine

## N,N′-Dimethyl-1,3-propanediamine (0.061 mL, 0.49 mmol) was added to a solution of 2-(2-(bromomethyl)phenyl)-4,4,5,5-tetramethyl-1,3,2-dioxaborolane (0.305 g, 1.03 mmol) and K2CO3 (0.203 g, 1.47 mmol)  in MeCN (5 mL) at 0 oC. The reaction was allowed to warm to rt and stir for 16 h before being filtered and concentrated in-vacuo to afford the title compound as a clear oil (0.24 g, 0.45 mmol, 92 %). 1H NMR (500 MHz, CDCl3)  7.59 (d, J = 6.8 Hz, 2 H, ArH), 7.26 - 7.18 (m, 4 H, ArH), 7.14 (d, J = 7.3 Hz, 2 H, ArH), 3.72 (s, 4 H, CH2Ar), 2.59 - 2.51 (m, 4 H, N(CH2CH2), 2.32 (s, 6 H, CH3N), 1.81 - 1.68 (m, 2H, N(CH2CH2CH2)N), 1.31 (s, 24 H, BPin); 13C NMR (125.5 MHz, CDCl3)  142.6, 133.1, 128.8, 126.6, 126.1, 82.0, 61.1, 54.5, 42.1, 25.7; I.R (thinfilm) ν max (cm-1): 2981.95 (C-H Sp3); HRMS (ESI): m/z calculated for C31H49B2N2O4: requires 535.3878 for [M+H]+, found 535.3869

N1, N6-dimethyl-N1, N6-bis(2-(4,4,5,5-tetramethyl-1,3,2-dioxaborolan-2-yl) benzyl)hexane-1,6-diamine

N1, N6-Dimethylhexane-1,6-diamine (0.25 mL, 1.40 mmol) was added dropwise to a solution of 2-(2-(bromomethyl)phenyl)-4,4,5,5-tetramethyl-1,3,2-dioxaborolane (0.873 g, 2.94 mmol) and K2CO3 (0.580 g, 4.20 mmol) in MeCN (10 mL) at 0 oC. The reaction was allowed to warm to rt and stir for 16 h before being filtered and concentrated in-vacuo to afford the crude material. The crude material was triturated with EtOAc to afford the title compound as a white solid (0.128 g, 0.22 mmol, 16 %). Mp 100-103 oC. 1H NMR (300 MHz, CDCl3)  7.56 (m, 2 H, ArH), 7.25 - 7.18 (m, 4 H, ArH), 7.08 (m, 2 H, ArH), 3.76 (s, 4 H, CH2Ar), 2.71 - 2.58 (m, 4 H, N(CH2CH2)), 2.40 (s, 6 H, CH3N), 1.63 - 1.45 (m, 4 H, N(CH2CH2)), 1.32 (s, 24 H, BPin), 1.25 (s, 4 H, N(CH2CH2); 13C NMR (75 MHz, CDCl3)  141.4, 132.3, 128.2, 126.9, 124.8, 81.3, 60.9, 55.6, 41.9, 27.3, 26.2, 24.9, 24.0; I.R (thinfilm) ν max (cm-1): 3039.8 (C-H sp2), 2962.07 (C-H Sp3); HRMS (ESI): m/z calculated for C34H55B2N2O4: requires 577.4348 for [M+H]+, found 577.4350

References

(1)	Bard, A. J. Electrogenerated Chemiluminescence; M. Dekker: New-York, 2004.

(2)	Rubinstein, I.; Bard, A. J. J. Am. Chem. Soc. 1981, 103, 512.

(3)	Deaver, D. R. Nature 1995, 377, 758.

(4)	Liu, Z.; Qi, W.; Xu, G. Chemical Society Reviews 2015, 44, 3117.

(5)	Blackburn, G. F.; Shah, H. P.; Kenten, J. H.; Leland, J.; Kamin, R. A.; Link, J.; Peterman, J.; Powell, M. J.; Shah, A.; Talley, D. B.; Tyagi, S. K.; Wilkins, E.; Wu, T.-G.; Massey, R. J. Clin. Chem. 1991, 37, 1534.

(6)	Yang, H.; Leland, J. K.; Yost, D.; Massey, R. J. Nat. Biotechnol. 1994, 12, 193.

(7)	Miao, W. Chem. Rev. 2008, 108, 2506.

(8)	Lai, R. Y.; Chiba, M.; Kitamura, N.; Bard, A. J. Anal. Chem. 2002, 74, 551.

(9)	Muegge, B. D.; Richter, M. M. Anal. Chem. 2002, 74, 547.

(10)	Schmittel, M.; Lin, H.-W. Angew. Chem. Int. Ed. 2007, 46, 893.

(11)	Lin, H.; Cinar, M. E.; Schmittel, M. Dalton Trans. 2010, 39, 5130.

(12)	Shu, Q.; Adam, C.; Sojic, N.; Schmittel, M. Analyst 2013, 138, 4500.

(13)	Berni, E.; Gosse, I.; Badocco, D.; Pastore, P.; Sojic, N.; Pinet, S. Chem. Eur. J. 2009, 15, 5145.

(14)	Jameison, F.; Sanchez, R. I.; Dong, L.; Leland, J. K.; Yost, D.; Martin, M. T. Anal. Chem. 1996, 68, 1298.

(15)	Liang, P.; Sanchez, R. I.; Martin, M. T. Anal. Chem. 1996, 68, 2426.

(16)	Sun, X. L.; Zhai, W. L.; Fossey, J. S.; James, T. D. Chemical Communications 2016, 52, 3456.

(17)	Zhai, W. L.; Sun, X. L.; James, T. D.; Fossey, J. S. Chemistry-an Asian Journal 2015, 10, 1836.

(18)	Sun, X. L.; James, T. D. Chemical Reviews 2015, 115, 8001.

(19)	Li, M.; Zhu, W. H.; Marken, F.; James, T. D. Chemical Communications 2015, 51, 14562.

(20)	Bull, S. D.; Davidson, M. G.; Van den Elsen, J. M. H.; Fossey, J. S.; Jenkins, A. T. A.; Jiang, Y. B.; Kubo, Y.; Marken, F.; Sakurai, K.; Zhao, J. Z.; James, T. D. Accounts of Chemical Research 2013, 46, 312.

(21)	Nishiyabu, R.; Kubo, Y.; James, T. D.; Fossey, J. S. Chemical Communications 2011, 47, 1124.

(22)	Nishiyabu, R.; Kubo, Y.; James, T. D.; Fossey, J. S. Chemical Communications 2011, 47, 1106.

(23)	Leland, J. K.; Powell, M. J. J. Electrochem. Soc. 1990, 137, 3127.

(24)	Kanoufi, F.; Zu, Y.; Bard, A. J. J. Phys. Chem. B 2001, 105, 210.

(25)	Liu, X.; Shi, L.; Niu, W.; Li, H.; Xu, G. Angew. Chem. Int. Ed. 2007, 46, 421.

(26)	Kanoufi, F.; Bard, A. J. J. Phys. Chem. B 1999, 103, 10469.

(27)	Kerr, E.; Doeven, E. H.; Wilson, D. J. D.; Hogan, C. F.; Francis, P. S. Analyst 2016, 141, 62.

(28)	Ritchie, E. L.; Pastore, P.; Wightman, R. M. J. Am. Chem. Soc. 1997, 119, 11920.

(29)	Wightman, R. M.; Forry, S. P.; Maus, R.; Badocco, D.; Pastore, P. J. Phys. Chem. B 2004, 108, 19119.

(30)	Badocco, D.; Zanon, F.; Pastore, P. Electrochim. Acta 2006, 51, 6442.

(31)	Miao, W.; Choi, J.-P.; Bard, A. J. J. Am. Chem. Soc. 2002, 124, 14478.

(32)	Sentic, M.; Milutinovic, M.; Kanoufi, F.; Manojlovic, D.; Arbault, S.; Sojic, N. Chem. Sci. 2014, 5, 2568.

(33)	Chen, X.-M.; Su, B.-Y.; Song, X.-H.; Chen, Q.-A.; Chen, X.; Wang, X.-R. TrAC Trends in Analytical Chemistry 2011, 30, 665.

(34)	Knight, A. W.; Greenway, G. M. Analyst 1996, 121, 101R.

(35)	Bock, C. R.; Connor, J. A.; Gutierrez, A. R.; Meyer, T. J.; Whitten, D. G.; Sullivan, B. P.; Nagle, J. K. J. Am. Chem. Soc. 1979, 101, 4815.

(36)	Lorand, J. P.; Edwards, J. O. The Journal of Organic Chemistry 1959, 24, 769.

(37)	Bosch, L. I.; Fyles, T. M.; James, T. D. Tetrahedron 2004, 60, 11175.

(38)	Smith, J. R. L.; Masheder, D. Journal of the Chemical Society, Perkin Transactions 2 1976, 47.

(39)	Masui, M.; Sayo, H.; Tsuda, Y. Journal of the Chemical Society B: Physical Organic 1968, 973.

(40)	Zu, Y.; Bard, A. J. Anal. Chem. 2000, 72, 3223.

(41)	Pastore, P.; Badocco, D.; Zanon, F. Electrochim. Acta 2006, 51, 5394.

(42)	Zhu, L.; Shabbir, S. H.; Gray, M.; Lynch, V. M.; Sorey, S.; Anslyn, E. V. Journal of the American Chemical Society 2006, 128, 1222.

(43)	Collins, B. E.; Sorey, S.; Hargrove, A. E.; Shabbir, S. H.; Lynch, V. M.; Anslyn, E. V. The Journal of Organic Chemistry 2009, 74, 4055.

(44)	James, T. D.; Sandanayake, K.; Iguchi, R.; Shinkai, S. Journal of the American Chemical Society 1995, 117, 8982.

(45)	James, T. D.; Sandanayake, K.; Shinkai, S. Angewandte Chemie-International Edition in English 1994, 33, 2207.

(46)	Arimori, S.; Bell, M. L.; Oh, C. S.; Frimat, K. A.; James, T. D. Journal of the Chemical Society-Perkin Transactions 1 2002, 803.

(47)	Arimori, S.; Bell, M. L.; Oh, C. S.; Frimat, K. A.; James, T. D. Chemical Communications 2001, 1836.

(48)	Phillips, M. D.; Fyles, T. M.; Barwell, N. P.; James, T. D. Chemical Communications 2009, 6557.

Supporting Information

N1, N6-dimethyl-N1, N6-bis(2-(4,4,5,5-tetramethyl-1,3,2-dioxaborolan-2-yl) benzyl)hexane-1,6-diamine (300 MHz, CDCl3)

N1, N6-dimethyl-N1, N6-bis(2-(4,4,5,5-tetramethyl-1,3,2-dioxaborolan-2-yl) benzyl)hexane-1,6-diamine (75.5 MHz, CDCl3)

N1,N3-dimethyl-N1,N3-bis(2-(4,4,5,5-tetramethyl-1,3,2-dioxaborolan-2-yl)benzyl)propane-1,3-diamine (500 MHz, CDCl3)

N1,N3-dimethyl-N1,N3-bis(2-(4,4,5,5-tetramethyl-1,3,2-dioxaborolan-2-yl)benzyl)propane-1,3-diamine (125.75 MHz, CDCl3)

Scheme S1. Synthesis of the boronic acid based coreactants PBA, ACS259 and ACS295.

Figure S1. Comparison of the (a) voltammetric and (b) ECL signals of Ru(bpy)32+ with either the coreactants TPrA (red line), PBA (blue line), ACS259 (black line) or ACS295 (green line). Experiments were performed in 100 mM PBS solution (pH 7.4) containing 10 µM Ru(bpy)32+ and 0.1 mM coreactant with a GC electrode as a working electrode. Scan rate 0.1 V s-1.

Figure S2. Effect of fructose addition on voltammetric and ECL responses. (a-b) Cyclic voltammetry and (c) ECL signal of a PBS solution (pH 7.4) containing 10 µM Ru(bpy)32+, 0.1 mM PBA and different concentrations of fructose. The arrow indicates increasing concentrations of fructose (0, 0.5, 1, 10, 50 mM). Experiments were performed on GC electrode at a scan rate of 0.1 V s-1.

Figure S3. Comparison of the PL and ECL spectra recorded in presence of increasing concentrations of fructose. Spectra were recorded in PBS (pH 7.4) solution with 10 µM Ru(bpy)32+, 0.1mM PBA and different concentrations of fructose (0, 0.5, 1, 10 and 50 mM). λexc= 452 nm.

Figure S4. Effect of fructose on voltammetric and ECL signals of the model tandem system Ru(bpy)32+/TPrA. (a) Cyclic voltammograms and (b) ECL signals of a PBS solution (pH 7.4) containing 10 µM Ru(bpy)32+, 0.1 mM TPrA before (red line) and after the addition of 10 mM fructose (blue line). Green line corresponds to the PBS solution containing 10 µM Ru(bpy)32+ and only 10 mM fructose (i.e. without TPrA or PBA). Experiments were performed on GC electrode at a scan rate of 0.1 V s-1.

Figure S5. Effect of (a-b) glucose or (c-d) fructose addition on electrochemical oxidation of ACS259 and on the corresponding ECL responses. Cyclic voltammograms and ECL signals of a PBS solution (pH 7.4) containing 10 µM Ru(bpy)32+, 0.1 mM ACS259 and different concentrations of glucose or of fructose. The arrow indicates increasing concentrations of glucose (0, 0.5, 1, 10, 50, 100 and 200 mM) or of fructose (0, 0.5, 1, 10 and 50 mM). Experiments were performed on glassy carbon (GC) electrode at a scan rate of 0.1 V s-1.

Figure S6. Variation of the ECL peak intensity (triangles) and of the oxidation peak current (dots) for the system Ru(bpy)32+/ACS259 as a function of the concentration of glucose (red markers) or of fructose (blue markers). Values are extracted from Figure S5.

Figure S7. Variation of the (a) voltammetric response and (b) ECL peak intensity for the system Ru(bpy)32+/ACS259 in a competitive assay. Cyclic voltammograms and ECL signals of a PBS solution (pH 7.4) containing 10 µM Ru(bpy)32+, 0.1 mM ACS259 (red curve) after adding first 10 mM glucose (blue curve) and then 10 mM fructose (green curve). Experiments were performed on glassy carbon (GC) electrode at a scan rate of 0.1 V s-1.

Figure S8. Effect of glucose addition on electrochemical oxidation of ACS295. Cyclic voltammograms of a PBS solution (pH 7.4) containing 10 µM Ru(bpy)32+, 0.1 mM ACS295 and different concentrations of glucose. The arrow indicates increasing concentrations of glucose (0, 0.5, 1, 10 and 50 mM). Experiments were performed on glassy carbon (GC) electrode at a scan rate of 0.1 V s-1.